SIMATS ENGINEERING

Department of Computer Science & Engineering

ASSESSMENT TOOL 1- ANALYTICAL PROBLEM SOLVING

| Course Code: | CSA1209 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO2 | Assessment: | ASSESSMENT TOOL1- ANALYTICAL PROBLEM SOLVING |
| Weightage: | 45% | Total Marks: | 25 |
| CO2 Statement: | Apply integer and floating-point arithmetic algorithms including Booth's multiplication, restoring/non-restoring division, and IEEE 754 standard operations to solve fixed-point and floating-point computational problems. (BL3) |  |  |
| Student Name: | K Sreelatha | Reg.No.: | 192425248 |
| Department: | AI & ML | Date: | 8/08/2026 |

ANNEXURE A

- A processor performs arithmetic operations on signed 8-bit integers using 2’s complement representation. Consider two numbers: +45 and −23. Analyze in detail how the system performs both addition and subtraction, including binary conversion, alignment of sign bits, and intermediate steps. Determine whether overflow occurs, and explain how the processor detects overflow and interprets the final result.

- A high-performance computing system replaces a ripple carry adder with a 4-bit carry look-ahead adder (CLA) to reduce computation delay. Given two binary inputs, analyze how generate and propagate signals are formed and how carries are computed in advance. Compare the time delay of CLA with ripple carry addition and justify, with reasoning, why CLA improves speed in modern processors.

- In a signed multiplication operation, a processor uses Booth’s algorithm to multiply two numbers, −6 and +5. Analyze the algorithm step-by-step, including the role of the accumulator, multiplier, and Booth bit, along with arithmetic shifts. Explain how the algorithm efficiently handles signed numbers and reduces the number of addition/subtraction operations compared to traditional multiplication.

- A processor is required to perform division of two binary numbers (e.g., 13 ÷ 3) using both restoring and non-restoring division algorithms. Analyze both methods step-by-step, showing intermediate remainders and quotient formation. Compare the two approaches in terms of number of steps, complexity, and efficiency, and explain why non-restoring division is often preferred in modern systems.

- A scientific application requires precise handling of real numbers using the IEEE 754 standard for floating-point representation. Given a decimal number (e.g., −13.25), analyze how it is represented in both single precision and double precision formats, including sign, exponent, and mantissa fields. Further, explain how floating-point addition is performed between two such numbers, highlighting issues like normalization, rounding, and precision loss.

Code of Conduct:

I __K Sreelatha(192425248)_ (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment. I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student: Sree K

MARKS ALLOCATION

|  | Problem Understanding (20%) | Method Selection (20%) | Solution Process (25%) | Accuracy of Calculation (20%) | Interpretation of Results (15%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 20% | Clearly interprets problem, identifies all parameters and requirements accurately | Understands problem with minor gaps | Partial understanding; misses key elements | Misinterprets or unable to understand problem |
| Method Selection | 20% | Selects most appropriate method/formula with strong justification | Selects correct method with minor errors | Inappropriate or partially correct method | Unable to select method |
| Solution Process | 25% | Logical, systematic steps with correct approach throughout | Mostly correct steps with minor mistakes | Disorganized steps; multiple errors | No clear process |
| Accuracy of Calculation | 20% | All calculations accurate; correct final answer | Minor calculation errors | Frequent errors; incorrect final answer | No valid calculations |
| Interpretation of Results | 15% | Clearly interprets results with units and engineering relevance | Basic interpretation with minor omissions | Limited or unclear interpretation | No interpretation |

1. Signed 8-bit Addition and Subtraction Using 2’s Complement

Given

- Number 1 = +45

- Number 2 = −23

- Representation = 8-bit 2’s complement

Step 1: Convert +45 to binary

45 in binary:

So,

+45 = 00101101

Step 2: Convert −23 to 2’s complement

First convert +23:

Take 1’s complement:

00010111

11101000

Add 1:

11101000

+      1

--------

11101001

Therefore,

−23 = 11101001

The leftmost bit is 1, indicating a negative number.

Addition: +45 + (−23)

00101101   (+45)

+ 11101001   (−23)

------------

00010110

There is a carry out of the MSB, which is discarded.

00010110 = 22

Therefore:

Overflow analysis

Overflow in 2's complement occurs when:

- two positive numbers produce a negative result, or

- two negative numbers produce a positive result.

Here, one number is positive and the other is negative.

Therefore, overflow cannot occur.

Another way to detect overflow is:

For this addition, the result is valid, so:

Final result

+45 + (−23) = +22

Subtraction: +45 − (−23)

Subtraction is performed using 2's complement:

The 2's complement of −23 is +23:

−23 = 11101001

Complement:

00010110

Add 1:

00010111 = +23

Now add:

00101101   (+45)

+ 00010111   (+23)

------------

01000100

01000100 is decimal 68.

Therefore:

Overflow

Both operands being added are positive:

68 is within the signed 8-bit range:

Therefore, no overflow occurs.

Final result

+45 − (−23) = +68

Conclusion

The processor uses 2's complement arithmetic for both addition and subtraction. Sign extension/alignment ensures that the operands occupy the same 8-bit format. Overflow is detected by examining the carry into and out of the sign bit or by checking whether two operands with the same sign produce a result with a different sign.

2. 4-bit Carry Look-Ahead Adder

A Carry Look-Ahead Adder (CLA) is designed to reduce the delay caused by carry propagation in a Ripple Carry Adder (RCA).

Consider two 4-bit numbers:

A = A3 A2 A1 A0

B = B3 B2 B1 B0

For each bit, two signals are generated.

Generate

A carry is generated when both input bits are 1.

Propagate

A carry is propagated when exactly one of the input bits is 1.

Carry equations

For bit 0:

For bit 1:

For bit 2:

For bit 3:

The sum is:

Example

Consider:

A = 1011

B = 0110

Starting from bit 0:

A = 1 0 1 1

B = 0 1 1 0

The CLA first calculates all P and G values.

P0 = A0 XOR B0

P1 = A1 XOR B1

P2 = A2 XOR B2

P3 = A3 XOR B3

Similarly:

G0 = A0 AND B0

G1 = A1 AND B1

G2 = A2 AND B2

G3 = A3 AND B3

The carry look-ahead logic then calculates C1, C2, C3, and C4 in parallel, rather than waiting for each carry to travel through the previous stage.

The result is:

For a 4-bit result:

Sum = 0001

Carry = 1

CLA vs Ripple Carry Adder

Ripple Carry Adder

In an RCA:

FA0 → FA1 → FA2 → FA3

The carry from one full adder must reach the next full adder.

Therefore, the delay increases approximately linearly with the number of bits:

Carry Look-Ahead Adder

CLA calculates carries simultaneously using Boolean equations.

Therefore:

for sufficiently large word sizes.

Why CLA is faster

In a ripple carry adder, the processor must wait for:

C0 → C1 → C2 → C3 → C4

In a CLA, the carries are calculated directly from the propagate and generate signals.

Therefore, CLA significantly reduces carry propagation delay and is useful in high-speed arithmetic units and modern processors.

Conclusion

CLA requires more hardware than RCA, but its parallel carry calculation provides much higher speed. This is why carry look-ahead techniques are important in high-performance processors.

3. Booth's Algorithm: Multiplication of −6 and +5

Given

Using 4-bit 2's complement:

−6 = 1010

+5 = 0101

Booth's algorithm uses:

- A → Accumulator

- Q → Multiplier

- M → Multiplicand

- Q−1 → Booth bit

Initially:

A   = 0000

Q   = 0101

M   = 1010

Q−1 = 0

Booth's decision table:

| Q0 Q−1 | Operation |
| --- | --- |
| 00 | No operation |
| 01 | A = A + M |
| 10 | A = A − M |
| 11 | No operation |

After the operation, perform an arithmetic right shift on:

A Q Q−1

Step 1

Initial:

A = 0000

Q = 0101

Q−1 = 0

Check:

Q0 Q−1 = 1 0

Therefore:

Since:

M = 1010

which represents −6,

subtracting M is equivalent to adding +6:

0000

- 1010

-------

0110

So:

A = 0110

Now perform arithmetic right shift:

A Q Q−1

0110 0101 0

After shift:

A = 0011

Q = 0010

Q−1 = 1

Step 2

Now:

Q0 Q−1 = 0 1

Therefore:

0011

+ 1010

-------

1101

So:

A = 1101

Arithmetic right shift:

A = 1110

Q = 1001

Q−1 = 0

Step 3

Now:

Q0 Q−1 = 1 0

Therefore:

1110

- 1010

-------

0100

Arithmetic right shift:

A = 0010

Q = 0100

Q−1 = 1

Step 4

Now:

Q0 Q−1 = 0 1

Therefore:

0010

+ 1010

-------

1100

Arithmetic right shift:

A = 1110

Q = 0010

Q−1 = 0

Final product:

A Q

1110 0010

Therefore:

11100010

This is a negative 8-bit number.

Find its magnitude:

11100010

invert → 00011101

add 1 → 00011110

00011110 = 30

Therefore:

Verification

Correct.

Why Booth's algorithm is efficient

Traditional binary multiplication may require an addition for every 1 in the multiplier.

Booth's algorithm examines pairs:

and replaces consecutive sequences of 1s with an addition and subtraction operation.

Therefore, it can reduce the number of arithmetic operations, particularly when the multiplier contains long runs of 1s.

It also naturally supports signed 2's complement numbers.

4. Restoring and Non-Restoring Division: 13 ÷ 3

Given:

Expected result:

Binary:

13 = 1101

3  = 0011

A. Restoring Division

Initial values:

A = 00000

Q = 1101

M = 00011

The algorithm performs one iteration for every dividend bit.

Step 1

Shift left:

A Q

00000 1101

↓

00001 1010

Subtract M:

00001

-00011

------

11110

Result is negative.

Therefore, restore A:

A = 00001

Set quotient bit:

Q0 = 0

Step 2

Shift left:

00001 1010

↓

00011 0100

Subtract:

00011

-00011

------

00000

Result is non-negative.

Therefore:

A = 00000

Q0 = 1

Step 3

Shift left:

00000 0100

↓

00000 1000

Subtract:

00000

-00011

------

11101

Negative, so restore:

A = 00000

Set:

Q0 = 0

Step 4

Shift left:

00000 1000

↓

00001 0000

Subtract:

00001

-00011

------

11110

Negative, so restore:

A = 00001

Q0 = 0

Final:

Q = 0100

A = 00001

Therefore:

B. Non-Restoring Division

The major difference is that when subtraction produces a negative remainder, the algorithm does not immediately restore it.

Instead, the sign is used to determine the operation in the next iteration.

Initial:

A = 00000

Q = 1101

M = 00011

Iteration 1

Shift:

A Q → 00001 1010

Subtract:

00001 - 00011 = 11110

Negative.

Set quotient bit:

Q0 = 0

Do not restore.

Iteration 2

Shift left:

11110 1010

→

11101 0100

Since A is negative, add M:

11101

+00011

------

00000

Positive.

Set:

Q0 = 1

Iteration 3

Shift:

00000 0100

→

00000 1000

Subtract M:

00000

-00011

------

11101

Negative.

Set:

Q0 = 0

Iteration 4

Shift:

11101 1000

→

11011 0000

A is negative, so add M:

11011

+00011

------

11110

Negative.

Set:

Q0 = 0

At the end, because A is negative, add M:

11110

+00011

------

00001

Thus:

Q = 0100

A = 00001

Therefore:

Comparison

| Feature | Restoring Division | Non-Restoring Division |
| --- | --- | --- |
| Negative remainder | Immediately restored | Not immediately restored |
| Addition/subtraction | More operations | Fewer operations |
| Hardware complexity | Relatively simple | More control logic |
| Speed | Slower | Faster |
| Efficiency | Lower | Higher |

Why non-restoring division is preferred

Restoring division may require an additional restoration operation whenever subtraction produces a negative result.

Non-restoring division avoids this immediate restoration and compensates during the next iteration.

Therefore, it generally requires fewer arithmetic operations and provides better performance, making it attractive for high-speed arithmetic hardware.

5. IEEE 754 Representation of −13.25

IEEE 754 represents floating-point numbers using:

Sign | Exponent | Fraction/Mantissa

For single precision:

1 bit | 8 bits | 23 bits

For double precision:

1 bit | 11 bits | 52 bits

Given:

Step 1: Convert 13.25 to binary

Integer part:

Fractional part:

Therefore:

Normalize:

Single Precision

Sign

The number is negative:

Exponent

Actual exponent:

Single precision bias:

Therefore:

130 in binary:

10000010

Fraction

From:

The leading 1 is implicit.

Fraction field:

10101000000000000000000

Therefore:

Sign     = 1

Exponent = 10000010

Fraction = 10101000000000000000000

Complete 32-bit representation:

1 10000010 10101000000000000000000

or:

11000001010101000000000000000000

Thus, −13.25 is represented exactly in IEEE 754 single precision because its binary fraction terminates.

Double Precision

Double precision has:

1 sign bit

11 exponent bits

52 fraction bits

Sign:

1

Actual exponent:

3

Double precision bias:

Therefore:

1026 in binary:

10000000010

Fraction:

10101000000000000000000000000000000000000000000000

Therefore:

Sign     = 1

Exponent = 10000000010

Fraction = 10101000000000000000000000000000000000000000000000

So the double-precision representation is:

1 | 10000000010 | 10101000000000000000000000000000000000000000000000

Floating-Point Addition

Suppose the processor needs to add two floating-point numbers:

and another number:

The processor performs several stages.

Step 1: Compare exponents

X exponent = 3

Y exponent = 2

The smaller exponent must be adjusted.

So:

Step 2: Add significands

1.101010

+ 0.101000

-----------

10.010010

Step 3: Normalize

The result is:

Normalize:

The exponent is increased from 3 to 4.

Step 4: Rounding

If the resulting fraction contains more bits than the available fraction field, the processor rounds it according to the selected IEEE 754 rounding mode.

The most common default is:

Round to nearest, ties to even.

Step 5: Store the result

The processor stores:

Sign

Exponent

Fraction

in the IEEE 754 format.