SIMATS ENGINEERING

Department of Computer Science & Engineering

ASSESSMENT TOOL 1- CONSTRAINT-BASED PROBLEM SOLVING

| Course Code: | CSA12 | Course Title: | Computer Architecture for Multicore Systems |
| --- | --- | --- | --- |
| CO Assessed: | CO5 | Assessment: | ASSESSMENT TOOL1- Constraint-Based Problem Solving |
| Weightage: | 40% | Total Marks: | 25 |
| CO5 Statement: | Construct I/O communication mechanisms by comparing memory-mapped and I/O-mapped techniques, interrupt handling (vectored, hardware, software), DMA controllers, and advanced bus interfaces including PCIe, USB, NVMe, and Thunderbolt. |  |  |
| Student Name: | K Sreelatha | Reg.No.: | 192425248 |
| Department: | AI & ML | Date: | 27/08/2026 |

ANNEXURE A

Constraint-Based Problem Solving

1.Construct an I/O communication mechanism for a real-time embedded system with the following constraints:

- CPU utilization must be below 15%

- Multiple sensors generate interrupts every 1 ms

- Use vectored interrupt handling

- Select between memory-mapped and I/O-mapped I/O for optimal latency

2.Design an I/O subsystem under these constraints:

- High-speed storage requires throughput > 3 GB/s

- Must use NVMe over PCIe

- CPU should not handle bulk data transfer

- Integrate DMA controller with interrupt notification

3.Construct a multi-device communication architecture with constraints:

- 6 peripherals using USB, 2 high-speed devices using Thunderbolt

- Avoid interrupt starvation

- Use hardware and software interrupts

- Ensure fair bus arbitration

4.Design a data acquisition system with constraints:

- Continuous data stream at 500 MB/s

- Minimal CPU intervention required

- Must compare DMA transfer vs interrupt-driven I/O

- Use memory-mapped I/O for device registers

5.Construct an interrupt handling mechanism with constraints:

- Support nested interrupts

- Maximum interrupt latency < 10 µs

- Use vectored interrupts for high-priority devices

- Use software interrupts for background tasks

Code of Conduct:

I _K Sreelatha(192425248)_ (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student: Sree K

MARKS ALLOCATION

|  | Problem Understanding (25%) | Constraint Analysis (25%) | Solution Development (25%) | Application of Concepts (15%) | Justification (10%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 25% | Clearly defines the problem and identifies all relevant constraints accurately | Identifies most constraints with minor gaps | Partial understanding; misses key constraints | Fails to identify constraints correctly |
| Constraint Analysis | 25% | Analyzes constraints effectively and prioritizes them logically | Provides reasonable analysis with some prioritization | Limited analysis; weak prioritization | No meaningful analysis or prioritization |
| Solution Development | 25% | Develops optimal and feasible solutions satisfying all constraints | Develops workable solutions with minor limitations | Solutions partially satisfy constraints | Solutions do not meet constraints |
| Application of Concepts | 15% | Effectively applies appropriate concepts, methods, or tools | Applies concepts with minor errors | Limited application; requires guidance | Unable to apply relevant concepts |
| Justification | 10% | Provides strong justification for decisions with logical reasoning | Provides justification with some clarity | Weak or unclear justification | No justification provided |

1. I/O Communication Mechanism for Real-Time Embedded System

Aim

To design an I/O communication mechanism for a real-time embedded system that supports multiple sensors with low CPU utilization and fast interrupt response.

Constraints

- CPU utilization < 15%

- Sensors generate interrupts every 1 ms

- Vectored interrupts required

- Low-latency I/O required

Architecture

Sensors

│

▼

┌──────────────────┐

│ Interrupt        │

│ Controller       │

└────────┬─────────┘

│

▼

CPU

│

System Bus

│

▼

DMA

│

▼

RAM

Working

- Sensors generate interrupts whenever new data is available.

- The vectored interrupt controller identifies the sensor and sends its vector to the CPU.

- The CPU executes the corresponding ISR.

- DMA transfers sensor data directly to RAM.

- CPU processes the data after receiving a DMA completion notification.

I/O Selection

Memory-Mapped I/O is selected because device registers can be accessed using normal load/store instructions and provide simple and efficient communication.

Result

Using MMIO + vectored interrupts + DMA reduces CPU workload and provides fast sensor communication while maintaining CPU utilization below 15%.

2. High-Speed I/O Subsystem Using NVMe over PCIe

Aim

To design a high-speed storage subsystem with throughput greater than 3 GB/s while minimizing CPU involvement.

Constraints

- Throughput > 3 GB/s

- NVMe over PCIe

- DMA-based transfer

- Interrupt notification

Architecture

CPU

│

▼

PCIe Controller

│

▼

NVMe SSD

│

▼

DMA

│

▼

RAM

Working

- CPU places a read/write command in the NVMe submission queue.

- NVMe controller processes the command.

- Data is transferred through PCIe.

- DMA transfers data directly between the NVMe device and RAM.

- After completion, an interrupt notifies the CPU.

- CPU processes the completion information.

Advantages

- High storage throughput

- Low CPU utilization

- Efficient large-data transfer

- Fast communication using PCIe

- DMA avoids unnecessary CPU data copying

Result

The NVMe + PCIe + DMA architecture provides high-speed storage communication and can achieve the required >3 GB/s throughput when sufficient PCIe bandwidth and suitable NVMe hardware are used.

3. Multi-Device Communication Architecture

Aim

To design a communication system for six USB peripherals and two high-speed Thunderbolt devices without interrupt starvation.

Architecture

CPU

│

┌────────▼────────┐

│ Interrupt       │

│ Controller      │

└───────┬─────────┘

│

┌───────┴────────┐

│                │

USB Controller   Thunderbolt

│             Controller

┌────┼────┐          │

USB1 USB2 ... USB6   TB1  TB2

Working

- USB devices communicate through the USB controller.

- Thunderbolt devices use the Thunderbolt controller.

- Hardware interrupts are used for immediate events.

- Software interrupts handle deferred/background processing.

- A priority mechanism handles urgent devices first.

- Round-robin arbitration provides fair bus access.

Avoiding Starvation

USB1 → USB2 → USB3 → USB4

→ USB5 → USB6 → TB1 → TB2

→ Repeat

Each device gets an opportunity to use the communication bus.

Result

Using hardware interrupts, software interrupts, priority handling, and round-robin arbitration ensures efficient communication and prevents interrupt starvation.

4. Data Acquisition System

Aim

To design a data acquisition system capable of continuously handling a 500 MB/s data stream with minimum CPU intervention.

Architecture

Data Acquisition Device

│

▼

MMIO Registers

│

▼

DMA

│

▼

Circular Buffer

(RAM)

│

▼

CPU

Working

- CPU configures the acquisition device using memory-mapped registers.

- DMA is configured with the memory address and transfer size.

- Data is continuously transferred from the device to RAM.

- DMA generates an interrupt after transferring a block.

- CPU processes the completed buffer while DMA continues receiving data.

DMA vs Interrupt-Driven I/O

| Feature | DMA | Interrupt I/O |
| --- | --- | --- |
| CPU involvement | Low | High |
| Large data transfer | Excellent | Less suitable |
| 500 MB/s stream | Suitable | Not ideal |
| CPU overhead | Low | High |
| Performance | High | Lower |

Result

DMA with memory-mapped I/O and a circular/double buffer is the best choice for a continuous 500 MB/s data stream because it reduces CPU intervention and maintains high throughput.

5. Interrupt Handling Mechanism

Aim

To design an interrupt handling mechanism supporting nested interrupts and low interrupt latency.

Constraints

- Nested interrupts

- Latency < 10 µs

- Vectored interrupts for high-priority devices

- Software interrupts for background tasks

Architecture

CPU

│

┌───────▼────────┐

│ Interrupt      │

│ Controller     │

└───────┬────────┘

│

┌──────┼──────┐

│      │      │

High   Medium   Low

Priority Priority Priority

│

▼

Vectored ISR

│

▼

Software Interrupt

│

▼

Background Tasks

Working

- The interrupt controller receives interrupt requests.

- High-priority devices are assigned specific interrupt vectors.

- The CPU directly jumps to the corresponding ISR.

- If a higher-priority interrupt occurs during a lower-priority ISR, it can interrupt the current ISR.

- After the high-priority ISR finishes, the previous ISR resumes.

- Software interrupts are used for non-critical background processing.

Reducing Latency

- Use vectored interrupts.

- Keep ISRs short.

- Assign high priority to critical devices.

- Support nested interrupts.

- Use DMA for large data transfers.

- Move lengthy operations to software interrupts.

Result

A priority-based vectored interrupt system with nested and software interrupts provides fast interrupt handling and is designed to meet the <10 µs latency requirement, subject to the actual processor and hardware implementation.